#include <TinyGPS++.h>

#include <HardwareSerial.h>

#include <SPI.h>

#include <LoRa.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

// =========================

// PATROL UNIT SETTINGS

// =========================

#define CAR_ID "CAR1"   // Change to CAR2 for second patrol car

// =========================

// WiFi & API SETTINGS

// =========================

const char* WIFI_SSID = "PNP-102";

const char* WIFI_PASSWORD = "1234567890";

const char* API_URL = "https://my-backend-874898295291.asia-southeast1.run.app/api/update-location";

// =========================

// GPS PINS

// =========================

#define GPS_RX 16

#define GPS_TX 17

#define GPS_BAUD 9600

// =========================

// LORA PINS

// =========================

#define LORA_SS   5

#define LORA_RST  14

#define LORA_DIO0 2

#define LORA_FREQ 433E6

// =========================

// OLED SETTINGS (128x32)

// =========================

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 32

#define OLED_ADDR 0x3C

#define OLED_SDA 21

#define OLED_SCL 22

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// =========================

// OBJECTS

// =========================

TinyGPSPlus gps;

HardwareSerial gpsSerial(2);

// =========================

// TIMING & CONNECTION

// =========================

unsigned long lastSendTime = 0;

const unsigned long sendInterval = 5000;

unsigned long popupUntil = 0;

String popupMessage = "BOOT";

bool gpsFixAnnounced = false;

// Connection Management

bool wifiConnected = false;

bool usingLoRa = false;

unsigned long lastWifiCheck = 0;

unsigned long wifiFailStartTime = 0;

const unsigned long wifiCheckInterval = 10000;

const unsigned long WIFI_FAIL_TIMEOUT = 20000; // 20 seconds before switching to LoRa

// =========================

// FUNCTIONS

// =========================

String buildPacket();

void sendLoRa(const String& message);

bool sendToAPI(const String& message);

void drawOLED();

void showPopup(const String& msg, unsigned long durationMs);

bool hasFixApprox();

String gpsStatusText();

void connectToWiFi();

void checkWiFiConnection();

String escapeJson(const String& input);

void setup() {

Serial.begin(115200);

gpsSerial.begin(GPS_BAUD, SERIAL_8N1, GPS_RX, GPS_TX);

Wire.begin(OLED_SDA, OLED_SCL);

if (!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {

Serial.println("SSD1306 init failed");

while (true);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(25, 10);

display.println("PATROL " CAR_ID);

display.display();

delay(1200);

// Connect to WiFi

connectToWiFi();

// Initialize LoRa (backup) - MUST MATCH RECEIVER SETTINGS

SPI.begin();

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(LORA_FREQ)) {

Serial.println("LoRa init failed!");

showPopup("LORA FAIL", 3000);

} else {

// MUST MATCH RECEIVER

LoRa.setTxPower(17);

LoRa.setSpreadingFactor(12);

LoRa.setSignalBandwidth(125E3);

LoRa.setCodingRate4(5);

LoRa.enableCrc();

Serial.println("LoRa initialized as backup");

}

Serial.println("Patrol transmitter started.");

Serial.println("WiFi primary (API), LoRa secondary (20s failover)");

showPopup("TX READY", 1500);

}

void loop() {

while (gpsSerial.available()) {

gps.encode(gpsSerial.read());

}

if (hasFixApprox() && !gpsFixAnnounced) {

gpsFixAnnounced = true;

Serial.println("GPS fix successful");

showPopup("FIX OK", 2000);

}

if (!gps.location.isValid()) {

gpsFixAnnounced = false;

}

// Check WiFi connection periodically

checkWiFiConnection();

if (millis() - lastSendTime >= sendInterval) {

lastSendTime = millis();

if (gps.location.isValid()) {

String packet = buildPacket();

bool sent = false;

// Try WiFi first (API) if not already in LoRa mode

if (!usingLoRa && WiFi.status() == WL_CONNECTED) {

sent = sendToAPI(packet);

if (sent) {

Serial.println("Sent via WiFi/API: " + packet);

showPopup("API SENT", 800);

// Reset fail timer on successful send

wifiFailStartTime = 0;

} else {

Serial.println("API send failed");

// Start fail timer if this is the first failure

if (wifiFailStartTime == 0) {

wifiFailStartTime = millis();

Serial.println("WiFi failure detected, will switch to LoRa after 20 seconds");

showPopup("WiFi ERR", 1200);

}

}

}

// Check if we should use LoRa (either already using it or WiFi failed for 20 seconds)

if (!sent && (usingLoRa || (wifiFailStartTime > 0 && (millis() - wifiFailStartTime >= WIFI_FAIL_TIMEOUT)))) {

if (!usingLoRa) {

// Switch to LoRa mode after 20 seconds

usingLoRa = true;

Serial.println("Switching to LoRa mode (WiFi failed for 20 seconds)");

showPopup("SWITCH LORA", 2000);

}

sendLoRa(packet);

Serial.println("Sent via LoRa: " + packet);

showPopup("LORA SENT", 800);

} else if (!sent && wifiFailStartTime > 0 && (millis() - wifiFailStartTime < WIFI_FAIL_TIMEOUT)) {

// Still in grace period, don't send anything

unsigned long remaining = (WIFI_FAIL_TIMEOUT - (millis() - wifiFailStartTime)) / 1000;

Serial.print("Waiting ");

Serial.print(remaining);

Serial.println(" seconds before switching to LoRa");

showPopup("WAIT " + String(remaining) + "s", 1000);

}

} else {

Serial.println("No valid GPS fix yet.");

showPopup("WAIT GPS", 1200);

}

}

drawOLED();

}

void connectToWiFi() {

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting to WiFi");

showPopup("WiFi CONN", 1500);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 20) {

delay(500);

Serial.print(".");

attempts++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\nWiFi connected!");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

showPopup("WiFi OK", 1500);

wifiConnected = true;

usingLoRa = false;

wifiFailStartTime = 0;

} else {

Serial.println("\nWiFi connection failed! Starting with LoRa.");

showPopup("WiFi FAIL", 1500);

wifiConnected = false;

usingLoRa = true;

}

}

void checkWiFiConnection() {

if (millis() - lastWifiCheck >= wifiCheckInterval) {

lastWifiCheck = millis();

if (WiFi.status() != WL_CONNECTED) {

if (wifiConnected) {

Serial.println("WiFi connection lost!");

showPopup("WiFi LOST", 1500);

wifiConnected = false;

// Start the 20 second timer

if (wifiFailStartTime == 0 && !usingLoRa) {

wifiFailStartTime = millis();

Serial.println("WiFi lost, will switch to LoRa after 20 seconds");

}

}

// Try to reconnect

WiFi.reconnect();

} else {

if (!wifiConnected) {

wifiConnected = true;

Serial.println("WiFi reconnected!");

showPopup("WiFi BACK", 1500);

// Reset LoRa mode and fail timer

usingLoRa = false;

wifiFailStartTime = 0;

}

}

}

}

bool sendToAPI(const String& message) {

if (WiFi.status() != WL_CONNECTED) {

Serial.println("WiFi not connected");

return false;

}

// Create JSON payload matching your receiver's format

String jsonPayload = "{";

jsonPayload += "\"packet\":\"" + escapeJson(message) + "\",";

jsonPayload += "\"source\":\"wifi_direct\",";

jsonPayload += "\"rssi\":0";

jsonPayload += "}";

Serial.println("Sending JSON to API: " + jsonPayload);

// Create secure client for HTTPS

WiFiClientSecure client;

client.setInsecure(); // Skip SSL verification for testing

HTTPClient http;

http.begin(client, API_URL);

http.setTimeout(10000);

http.addHeader("Content-Type", "application/json");

http.addHeader("Connection", "close");

int httpResponseCode = http.POST(jsonPayload);

if (httpResponseCode > 0) {

Serial.print("HTTP Response code: ");

Serial.println(httpResponseCode);

String response = http.getString();

Serial.println("Response: " + response);

http.end();

return (httpResponseCode == 200);

} else {

Serial.print("Error on HTTP request: ");

Serial.println(httpResponseCode);

http.end();

return false;

}

}

String buildPacket() {

double lat = gps.location.lat();

double lng = gps.location.lng();

double speedKmph = 0.0;

if (gps.speed.isValid()) {

speedKmph = gps.speed.kmph();

if (speedKmph < 3.0) speedKmph = 0.0;

}

int sats = gps.satellites.isValid() ? gps.satellites.value() : 0;

String timestamp = "0";

if (gps.date.isValid() && gps.time.isValid()) {

char buf[25];

snprintf(

buf, sizeof(buf),

"%04d-%02d-%02d %02d:%02d:%02d",

gps.date.year(),

gps.date.month(),

gps.date.day(),

gps.time.hour(),

gps.time.minute(),

gps.time.second()

);

timestamp = String(buf);

}

// 6-field format (MATCHES RECEIVER EXPECTATION):

// CAR1,lat,lng,speed,sats,timestamp

String packet = String(CAR_ID) + "," +

String(lat, 6) + "," +

String(lng, 6) + "," +

String(speedKmph, 2) + "," +

String(sats) + "," +

timestamp;

return packet;

}

void sendLoRa(const String& message) {

LoRa.beginPacket();

LoRa.print(message);

LoRa.endPacket();

}

bool hasFixApprox() {

int sats = gps.satellites.isValid() ? gps.satellites.value() : 0;

return gps.location.isValid() && sats >= 4;

}

String gpsStatusText() {

if (!gps.location.isValid()) return "NO FIX";

if (hasFixApprox()) return "FIX OK";

return "WEAK";

}

void showPopup(const String& msg, unsigned long durationMs) {

popupMessage = msg;

popupUntil = millis() + durationMs;

}

String escapeJson(const String& input) {

String out = "";

for (size_t i = 0; i < input.length(); i++) {

char c = input[i];

if (c == '\"') out += "\\\"";

else if (c == '\\') out += "\\\\";

else if (c == '\n') out += "\\n";

else if (c == '\r') out += "\\r";

else if (c == '\t') out += "\\t";

else out += c;

}

return out;

}

void drawOLED() {

int sats = gps.satellites.isValid() ? gps.satellites.value() : 0;

double speedKmph = gps.speed.isValid() ? gps.speed.kmph() : 0.0;

if (speedKmph < 3.0) speedKmph = 0.0;

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(0, 0);

display.print(CAR_ID);

// Show connection status

display.setCursor(32, 0);

if (usingLoRa) {

display.print("LoRa");

} else if (WiFi.status() == WL_CONNECTED) {

display.print("WiFi");

} else {

display.print("FAIL");

}

display.setCursor(68, 0);

display.print("SAT:");

display.print(sats);

display.setCursor(108, 0);

display.print((int)speedKmph);

display.print("K");

display.setCursor(0, 10);

display.print("GPS:");

display.print(gpsStatusText());

display.setCursor(0, 20);

if (millis() < popupUntil) {

display.print("MSG:");

display.print(popupMessage);

} else {

if (gps.location.isValid()) {

display.print(gps.location.lat(), 2);

display.print(",");

display.print(gps.location.lng(), 2);

} else {

display.print("LAT/LNG: --");

}

}

display.display();

}